Assessment

N+ - Network security, Maintenance and Troubleshooting procedures

Section A: Scenario-Based Concept Application

1.Explain the selection process for CPU, RAM, and storage specifications to ensure hardware compatibility and prevent performance bottlenecks for a startup team performing concurrent development and video conferencing tasks.

Ans.

1. The CPU (The Processor)

The CPU is the heart of the system. When you are working on development and multitasking you need to look at the CPU. You need to think about how cores the CPU has and what kind of architecture it uses.

The Baseline for the CPU is that you should have least 8 cores and 16 threads. For example you can use the AMD Ryzen 7 or the Intel Core i7 or the Apple M-series Pro chips.

Why the CPU matters for this workflow is that you do a lot of things at the time. The CPU is important for Concurrent Development. When you are compiling code or running Docker containers or spinning up virtual machines you are using a lot of CPU cores.

The CPU is also important for Video Conferencing. When you use apps like Zoom or Teams they use a lot of CPU cycles for real-time video encoding and background blurring and noise cancellation.

If your CPU does not have cores it can cause problems. For example if you are screen-sharing a development build during a meeting it can cause choppy video and audio dropouts and massive system lag. The CPU is the CPU. It is very important.

2. RAM (System Memory)

RAM is like a short-term workspace for the system. If you run out of RAM the system will start using the storage drive, which's very slow.

The Baseline for RAM is 32 GB. This is the spot for professional developers. If you have 16 GB it is not enough for development and video calls.

You should also think about the speed and compatibility of the RAM. You should look for DDR5 RAM that's at least 5200MHz.

It is also important to ensure synergy between the RAM and the CPU. You should buy RAM in pairs like two 16GB sticks of one 32GB stick. This is called a Dual-Channel Configuration. It doubles the memory bandwidth, which allows the CPU to access data faster. The. The RAM work together.

You should also make sure that the RAM speed and generation are supported by your chosen CPU and motherboard. The CPU and the motherboard and the RAM all need to work

3. Storage (SSD)

You should not use Hard Drives for active work. You should use Solid State Drives instead.

The Baseline for Storage is an NVMe M.2 SSD that uses PCIe Gen 4 or Gen 5. You should look for read. Write speeds that are, above 5,000 MB/s.

Why Storage matters for this workflow is that developers are always reading and writing thousands of small files. They are fetching dependencies and compiling code and saving assets. If the Storage is slow it slows down the pipeline. The Storage is very important.

If your CPU and RAM are very fast but your Storage is slow it can cause problems. The CPU will stall while waiting for data to load. This is called an I/O bottleneck. The Storage and the CPU and the RAM all need to work. The Storage is the Storage. It is very important.

2. Explain how system resource allocation and hardware bottlenecks contribute to performance degradation after the deployment of resource-intensive background applications like antivirus software

Ans.

1. The CPU Bottleneck: Context Switching and Interrupts

The processor gets slowed down. It is not just because the antivirus software is using some of the CPU cycles. It is about how the computer manages these cycles.

The operating system has to switch between the applications you are using and the antivirus software that is running in the background. Every time the computer switches between these tasks it has to save what the current application is doing and load what the antivirus software is doing. This takes a lot of time. Slows down the computer.

When the antivirus software starts running it gets rid of the data from the application you are using from the cache. The cache is a part of the computer that stores data so it can be accessed quickly. When the operating system switches back to your application the computer has to get the data from the memory, which's much slower.

The antivirus software also interrupts the computer when it is doing something. This happens when the antivirus software is checking files. Every time an application reads or writes a file the antivirus software stops the application. Checks the file.

2. The Storage Bottleneck: I/O Saturation and Queue Depth

The storage is usually the part of the computer. This makes it the part that gets slowed down the most.

When the antivirus software is scanning files it uses up a lot of the storages ability to read and write files. Every storage has a limit to how files it can read and write per second. When the antivirus software is using up all of this your application has to wait.

When the antivirus software is using the storage a lot it makes the storages queue get longer. Your application has to wait in line to use the storage, which makes it take longer to save files or get data.

With fast storage the antivirus software can still slow it down. This happens when the antivirus software is writing a lot of data to the storage. The storage has to get rid of data to make room for new data, which takes time.

3. The Memory Bottleneck: RAM Allocation and Paging

The memory gets slowed down when the operating system has to make room for the antivirus software.

The operating system tries to keep memory for each application. When the antivirus software needs memory the operating system may take some memory away from your application.

When the computer runs out of memory it uses the storage to store data. When your application needs data that is stored on the storage it takes a lot longer to get it. This is because getting data from the storage is much slower than getting it from the memory.

The operating system also uses some of the memory to store files that are used a lot. When the antivirus software scans files it gets rid of the files that your application needs from the memory. This makes your application have to get the files, from the storage, which takes longer.

3. Explain the systematic troubleshooting process for resolving peripheral hardware failures caused by driver incompatibility or operating system conflicts when a device functions inconsistently across different workstations.

Ans.

Phase 1: Isolation and Baseline Verification

Before start changing software configurations need to make sure we have a baseline to compare. This is to prove that the hardware works and to find out where the problem is. The problem might be with the workstation.

The Cross-Test: We should connect the peripheral to a machine that we know is working properly and has the same operating system version. If it works then the peripheral hardware and firmware are working correctly.

The Port & Cable Check: We should move the peripheral to a port and replace the cable. This will help we find out if the problem is with the interface or the power delivery from the port.

Isolate Through a Topology Diagram: We should make a diagram to visualize where the problem is happening. If the device is not working on ports on just one machine then the problem is probably with the operating system or driver stack.

Phase 2: Driver Triage and Clean Installation

If the hardware is working properly then the problem is probably with the driver. The driver might be corrupted, outdated or not the right one.

Step 1: Inspect Device Manager: We should look at the Device Manager on Windows or System Information on macOS. Check for exclamation marks that indicate a generic driver or a device that is not working properly.

Step 2: Perform a Clean Driver Uninstall: Do not just click "Update Driver". You should uninstall the device completely.

We can use tools like Device Cleanup Tool. Enable "Show hidden devices" in Device Manager to remove old drivers that might be causing problems.

Step 3: Deploy the Correct Driver State: We should download the driver for your workstations architecture. Install it using privileges.

Phase 3: Resolving OS and Kernel-Level Conflicts

If a clean driver install does not work then the problem is probably with the operating systems kernel or power-saving features.

1. OS Update Histories: If the peripheral was working last week but is not working now you should check the workstations update history.

The problem might be with an OS quality update or security patch that changed the kernels hardware abstraction layer.

The fix is to roll back the quality update to see if the peripheral starts working again.

2. Disable Aggressive Power Management: Modern operating systems turn off power to peripherals to save energy, which can cause problems.

You should open Device Manager locate the USB Root Hubs and uncheck "Allow the computer to turn off this device to save power".

You can also disable USB suspend in the advanced Windows Power Options.

3. Check for Shared Resources: Sometimes two devices can compete for the Interrupt Request line or memory address base.

You should open System Information. Navigate to Hardware Resources to check for conflicts.

Phase 4: Environmental and Software Layer Isolation

If the OS kernel is not the problem then you should check for background software conflicts.

You should execute a Clean Boot or Safe Mode: Reboot the workstation into Safe Mode or a Clean Boot with all non-Microsoft services disabled.

If the device works then a third-party background application is probably intercepting the peripherals data stream.

You should check the USB Controller/Chipset Drivers: Sometimes the problem is not, with the peripherals driver. With the motherboards USB controller driver.

You should update the AMD or Intel Chipset drivers for the malfunctioning workstation.

4. Explain the role of Uninterruptible Power Supply (UPS) systems and surge protection in preventing file system corruption and ensuring data integrity during unexpected power fluctuations.

Ans.

1. The UPS: Saving the File System

When the power goes out the computer system goes dark away causing two big problems:

Torn Writes: If the computer is moving a file from the storage area to the hard drive and the power goes out the file gets messed up. It does not get written completely. It becomes a mess that the computer can not read.

Corrupted Map Tables: The Solid-State Drives use a map to keep track of where the data is stored on the drive. If the power goes out this map can get corrupted. This means the data is still on the drive. The drive can not find it so the drive is useless.

How the UPS Fixes It

- The UPS finds out the power is out quickly and switches to its own battery power right away. Then it sends a signal to the computer.

- The computer gets this signal. Does the following things:

- It stops doing any new work.

- It makes sure all the data that is stuck in the storage area gets written to the hard drive.

- It closes everything down safely. Shuts off the computer.

2. Surge Protection: Saving the Physical Hardware

A blackout means there is no power. A surge means there is too much power. This can happen when there is lightning or a problem with the power grid. It sends a surge of power through the outlet.

Frying the Brain: This high power can burn out the parts of the hard drives and Solid-State Drives. If the main chip, on a drive gets burned out the drive stops working away.

Bit Rot: Sometimes the surges are not big enough to destroy the computer right away but they can still cause problems. They can make the hardware wear out over time which can cause the drive to make mistakes and change the data. This can make the files get corrupted so they will not open anymore. The UPS and the computer system and the Solid-State Drives and the hard drives are all important. The UPS helps to protect them. The UPS helps to save the file system and the physical hardware like the drives and the Solid-State Drives.

Section B: Practical Application

Task 1 : Physically assemble a workstation and document the component installation sequence.

Ans.

Phase 1: Outside the Case (Motherboard Prep)

- Step 1: Install the CPU. Lift the socket lever, align the golden triangles on the corner of the processor and the socket, drop it in gently, and lock the lever down.

- Step 2: Install the RAM. Open the clips on the slots (usually slots 2 and 4), line up the notch on the bottom of the RAM stick, and push down firmly until you hear a "click."

- Step 3: Install the M.2 SSD. Slide the tiny storage drive into its slot angle, push it flat, screw it down, and reinstall its metal heat shield.

Phase 2: Preparing the Case

- Step 4: Install the Power Supply (PSU). Slide the PSU into the bottom of the case with its fan facing downward (toward the bottom dust filter). Screw it in tight from the back of the case.

- Step 5: Prep the Case Standoffs. Make sure the gold standoff screws inside the case line up with the size of your motherboard so the board doesn't short-circuit against the bare metal.

Phase 3: Putting it All Together

- Step 6: Mount the Motherboard. Lower the motherboard into the case, align its ports with the rear opening, and screw it down to the standoffs.

- Step 7: Install the CPU Cooler. Apply a pea-sized drop of thermal paste to the CPU, mount the cooler on top, tighten the screws in an "X" pattern, and plug its wire into the cpu fan header.

- Step 8: Plug in the Cables. Connect the big motherboard power cables and the tiny front-panel wires (power button, USB ports, audio) before things get crowded.

- Step 9: Install the Graphics Card (GPU). Remove the metal slots on the back of the case, push the GPU into the top slot until it clicks, screw it to the case frame, and plug in its power cables.

Task 2 : Troubleshoot and resolve a RAM mismatch error within the BIOS/UEFI settings.

Ans.

Step 1: Turn Off Overclocking (XMP / EXPO)

When you mix RAM, old automated overclocking profiles will crash the system.

- Turn on the PC and mash Delete or F2 to enter the BIOS.

- Find the memory settings menu (called Ai Tweaker, Overclocking, or MIT).

- Look for XMP (Intel) or EXPO / DOCP (AMD) and switch it to Disabled or Auto.

Step 2: Set the Lowest Common Denominator

If "Auto" doesn't work, you must manually force the faster RAM stick to slow down and match the weaker stick.

- Set Speed: Find DRAM Frequency and set it to match your slowest stick (e.g., lock it to 2666MHz).

- Set Voltage: Find DRAM Voltage and set it manually (usually 1.35V or 1.20V) so both sticks get enough power.

- Set Timings: Open DRAM Timing Control and type in the higher, slower timing numbers of the weaker stick (e.g., change CL16 to CL18).

Step 3: Save and Exit

- Press F10 on your keyboard to save your changes and restart.

- The motherboard will now treat both sticks identically, resolving the mismatch error and allowing the computer to boot

Task 3 : Build a LAN topology in Packet Tracer for 5 systems and 1 printer using a Class C subnet.

Ans.

| Device | Type | Switch Port | Static IP Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| PC-0 | Workstation | Fa0/1 | 192.168.1.10 | 255.255.255.0 |
| PC-1 | Workstation | Fa0/2 | 192.168.1.11 | 255.255.255.0 |
| PC-2 | Workstation | Fa0/3 | 192.168.1.12 | 255.255.255.0 |
| PC-3 | Workstation | Fa0/4 | 192.168.1.13 | 255.255.255.0 |
| PC-4 | Workstation | Fa0/5 | 192.168.1.14 | 255.255.255.0 |
| Printer-0 | Printer Network | Fa0/6 | 192.168.1.15 | 255.255.255.0 |

2. Quick Setup Steps

- Step 1: Place Hardware: Drag a 2960 Switch into the center of the workspace. Surround it with 5 PCs and 1 Printer from the "End Devices" menu.

- Step 2: Connect Cables: Use the solid black Copper Straight-Through cable to connect each PC's FastEthernet0 port and the printer's FastEthernet0 port directly to the switch ports (Fa0/1 through Fa0/6). Wait for the amber lights to turn green.

- Step 3: Assign IP Addresses: * For PCs: Click a PC, Desktop, IP Configuration. Type in its specific IP and set the Subnet Mask to 255.255.255.0.

  - For the Printer: Click the Printer, Config tab, FastEthernet0. Type in its IP and Subnet Mask.

- Step 4: Test Connection: Click on PC-0, Desktop, Command Prompt. Type ping 192.168.1.50 and press Enter. If you get 4 successful replies, your LAN is working perfectly.

Task 4 : Perform a clean OS installation with manual disk partitioning and startup optimization.

Ans.

Phase 1: Clean OS Installation & Manual Partitioning

Step 1: Boot from the Installation USB

- Insert your bootable USB drive into a fast USB 3.0 port on the workstation.

- Power on the machine and repeatedly tap your motherboard's boot menu key (usually F12, F11, or F8).

- Choose the option prefixed with UEFI: [Your USB Drive Name] to launch the installer in modern UEFI mode.

Step 2: Navigate to Custom Setup

- Select your language, time format, and keyboard layout, then click Next.

- Click Install Now.

- When prompted for your setup type, choose Custom: Install Windows/Linux only (advanced). This prevents the system from automatically choosing your disk layouts.

Step 3: Delete Existing Partitions

- You will see a list of your hard drives and their current partitions.

- Click on each existing partition on your primary drive and click Delete until you are left with a single entry called Drive 0 Unallocated Space.

Step 4: Create the Manual Partitions

Click New (or Create) on the unallocated space to manually build a split-partition framework:

- Create the EFI Boot Partition: Click New, set the size to 500 MB, and click Apply. (The installer will automatically format this as FAT32 to store your system boot files).

- Create the System (C:) Drive: Select the remaining unallocated space, click New, set the size between 102400 MB (100 GB) and 153600 MB (150 GB), and click Apply. This zone is dedicated strictly to the OS and core software.

- Create the Data (D:) Drive: Select the last block of unallocated space, click New, leave the size at its maximum remaining value, and click Apply. This isolates your project files and personal data from the operating system.

Step 5: Begin Installation

- Click on the System (C:) partition you just created to highlight it.

- Click Next to begin installing the operating system files onto that specific partition. The PC will reboot automatically when finished.

Phase 2: Post-Installation Startup Optimization

Step 6: Trim Startup Applications

- Once you boot to your new desktop, press Ctrl + Shift + Esc to open the Task Manager.

- Click on the Startup Apps tab on the left-hand menu.

- Look through the list for non-essential software (such as browser pre-loaders, chat apps, or cloud storage syncing tools).

- Right-click each high-impact, non-essential app and select Disable. This stops them from launching automatically and consuming CPU cycles at boot time.

Step 7: Cut the Boot Screen Timeout Delay

- Press the Windows Key + R to open the Run dialog box.

- Type SystemPropertiesAdvanced and press Enter.

- Under the Startup and Recovery section at the bottom, click Settings.

- Locate "Time to display list of operating systems." Change this value from the default 30 seconds down to 3 seconds. Click OK.

Step 8: Verify Fast Startup

- Open the Control Panel and navigate to Power Options,Choose what the power buttons do.

- Click “Change settings that are currently unavailable” at the top.

- Ensure the box next to Turn on fast startup (recommended) is checked, then save changes. This enables a hybrid hibernation state that allows your PC to boot almost instantly.

Section C: Integrated Case Study

1.Identify the three hardware components most likely to cause random reboots under load.

Ans.

1. The Power Supply Unit (PSU)

- The Problem: Heavy loads make your CPU and graphics card demand a lot of electrical power.

- The Trigger: If your PSU is too weak, old, or low-quality, it cannot supply enough stable power. To prevent a dangerous electrical short or a fire, the PSU's built-in safety switches instantly cut the power.

- The Symptom: Your PC instantly goes black and restarts with zero warning—exactly as if someone yanked the power cord out of the wall.

2. The Central Processing Unit (CPU)

- The Problem: The CPU is the main brain of the computer. Under a heavy load, it generates massive amounts of heat.

- The Trigger: If your CPU cooler is broken, poorly attached, or the thermal paste has dried up, the CPU will rapidly overheat. Once it hits its maximum safe limit (usually around 95°C to 105°C), the motherboard forces a reboot to save the chip from melting.

- The Symptom: You might notice your computer lagging or stuttering for a few seconds right before it suddenly reboots.

3. The Graphics Processing Unit (GPU)

- The Problem: The graphics card handles heavy 3D rendering and gaming. Like the CPU, it gets hot and pulls massive amounts of power.

- The Trigger: If the GPU gets too hot, or if it pulls a sudden, microsecond-long burst of extreme power (called a transient spike) that shocks the system, safety sensors trip.

- The Symptom: Your monitor screen might instantly go black, or your graphics card fans will suddenly spin at 100% maximum speed, followed immediately by a system restart.

2.Explain why the systems pass POST in the morning but fail in the afternoon.

Ans.

1.Parts Getting Loose

What happens: When parts get hot during use and cool down at night they move a bit. This movement can cause RAM or graphics cards to slowly come loose from their slots.

Why does it happen in the afternoon? In the morning the electrical pins are just touching so the computer starts up. By the afternoon the heat has moved the parts enough to break the connection causing a problem.

2. Small Cracks in the Motherboard

What happens: stressed motherboards can get small cracks in their tiny electrical paths.

Why does it happen in the afternoon? When the PC is cold in the morning the metal sides of the crack are close together allowing electricity to flow. In the afternoon heat the motherboard. Expands a bit pulling the crack apart and breaking the circuit.

3. Hot Afternoon Room Temperature

What happens: Computers use the room air to stay cool. Morning air is usually cool. Afternoon air is much warmer because of the sun and electronics running all day.

Why does it happen in the afternoon? In the afternoon the inside of the PC case is already hot. The moment you turn it on a critical part, like the CPU or power supply gets too hot. The system stops the boot process to protect itself.

3. How would you use a stress-test tool to confirm a thermal or power issue?

Ans

1. Testing for Heat (Thermal Issue)

Run a CPU-only test, then a GPU-only test, and watch your temperatures.

- The Tell: Your temperatures steadily climb and shoot past 90°C–95°C. The fans will sound like a jet engine, the PC might start lagging heavily, and then it suddenly restarts.

- The Verdict: Your cooling is broken. You need to clean out dust, fix a broken fan/pump, or reapply thermal paste.

2. Testing for Power (PSU Issue)

Run the dedicated "Power" test in OCCT. This forces both your CPU and GPU to draw maximum electricity at the exact same time, putting a massive load on the Power Supply.

- The Tell: The PC instantly goes black and restarts the exact millisecond you click "Start Test"—before the components even have time to get hot.

- The Verdict: Your Power Supply Unit (PSU) is failing, old, or too weak to handle your hardware when it is working hard. You need to replace the PSU

4. Propose three specific steps to remediate the frequent system restarts.

Ans.

1. Refresh the Cooling (For Heat Issues)

If the PC crashed due to high temperatures, your cooling system needs maintenance.

- Action: Remove the CPU cooler, clean off the old crusty paste with isopropyl alcohol, apply a fresh pea-sized drop of new thermal paste, and screw the cooler back down evenly. Also, use compressed air to blow all dust out of the fans and radiators.

- Result: Restores proper heat transfer so the CPU stays well below its safety limit.

2. Upgrade the Power Supply (For Power Issues)

If the PC shut off instantly the moment a heavy test started, the PSU cannot handle the electrical load.

- Action: Replace the old Power Supply Unit (PSU) with a high-quality model from a reputable brand. Choose one with a higher wattage rating than your current system requires to safely handle sudden power spikes.

- Result: Ensures the computer gets a steady, unbroken stream of electricity when working hard.

3. Reseat All Major Components (For "Afternoon" Failures)

If the PC boots fine when it is cold but fails when it warms up, parts have likely wiggled loose due to thermal expansion.

- Action: Open the case, completely pull out the RAM sticks and the graphics card, and click them firmly back into their slots. Unplug and securely re-plug the main power cables on the motherboard.

- Result: Cleans the electrical contacts and locks the parts tightly in place so minor heat adjustments won't break the connection.

5. Provide a brief technical justification for why a PSU upgrade might (or might not) be necessary.

Ans.

Step 1: Parts Age: Inside a PSU, components called capacitors degrade over time from heat. An aging PSU loses its ability to hold a steady charge, meaning it can no longer deliver its original advertised wattage under load.

Step 2: Parts Demand Sudden Power: Modern graphics cards draw power in aggressive, split-second spikes (transient spikes) that can briefly double their normal energy demand.

Step 3: Voltage Drops: When a weak or aging PSU is hit with a sudden power spike, its voltage sags. The motherboard instantly detects this unstable power and forces a hard reset to protect its delicate digital circuits.

Step 4: Safety Switches Trip: If your components demand more total electricity than the PSU can safely handle, the PSU's built-in emergency switches (like Over-Current Protection) trip to cut the power and prevent a fire.