# Assignment: Ethernet Standards and Specification

## RJ-45 Connector Pin Layout

| Pin | T568A | T568B |
| --- | --- | --- |
| 1 | White/Green | White/Orange |
| 2 | Green | Orange |
| 3 | White/Orange | White/Green |
| 4 | Blue | Blue |
| 5 | White/Blue | White/Blue |
| 6 | Orange | Green |
| 7 | White/Brown | White/Brown |
| 8 | Brown | Brown |

T568A uses the green pair on pins 1-2 while T568B uses the orange pair on pins 1-2. Both provide identical performance.

Insert the RJ-45 T568A/T568B diagram image from the chat here.

## 2. Ethernet Standards Comparison

| Feature | 10BASE-T | 100BASE-TX | 1000BASE-T |  |
| --- | --- | --- | --- | --- |
| Speed | 10 Mbps | 100 Mbps | 1000 Mbps |  |
| Cable | Cat3/Cat5 | Cat5+ | Cat5e/Cat6+ |  |
| Max Length | 100 m | 100 m | 100 m |  |
| Connector | RJ-45 | RJ-45 | RJ-45 |  |

## 3. Connecting Two Laptops

Traditionally, use a crossover Ethernet cable because it swaps the transmit and receive pairs, allowing two similar devices to communicate directly. Modern laptops usually support Auto-MDI/MDIX, so either cable may work, but crossover is the standard answer.

## 4. Devices Using Ethernet

| Device | Likely Standard | Reason |
| --- | --- | --- |
| Desktop PC | 1000BASE-T | High-speed networking |
| Wi-Fi Router | 1000BASE-T | Gigabit LAN ports |
| Network Printer | 100BASE-TX | Sufficient bandwidth |