# Assignment: Day-3 IP Addressing

## 1. IPv4 vs IPv6

| IPv4 | IPv6 |
| --- | --- |
| 32-bit, dotted decimal | 128-bit, hexadecimal |
| ~4.3 billion addresses | Huge address space |
| Supports NAT commonly | NAT usually unnecessary |
| Header is simpler but less extensible | Improved header and auto-configuration |

## 2. Static IP Assignment

| Device | IP | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Phone | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| Smart TV | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| Printer | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |

## 3. /27 Subnetting

Borrowed bits: 3
Valid subnets: 8
Hosts per subnet: 30

1) 192.168.1.0 - 192.168.1.31 | Hosts: 192.168.1.1 - 192.168.1.30
2) 192.168.1.32 - 192.168.1.63 | Hosts: 192.168.1.33 - 192.168.1.62
3) 192.168.1.64 - 192.168.1.95 | Hosts: 192.168.1.65 - 192.168.1.94

## 4. IP Calculation

IP: 10.0.5.17
Subnet Mask: 255.255.255.0 (/24)
Network: 10.0.5.0
Broadcast: 10.0.5.255
Valid Hosts: 10.0.5.1 - 10.0.5.254

## 5. Verification

Verification Statement:

The results from the online subnet calculator matched my manual calculation exactly. Both showed the same network address, broadcast address, usable host range, and CIDR notation.